MATERIAL SAFETY DATA SHEET

AZ(R) 300 MIF DEVELOPER

| Substance key: BBG70N4 | REVISION DATE: 07/25/2005 |
| --- | --- |
| Version | Print Date: 07/25/2005 |

Section 01 - Product Information

Identification of the	AZ Electronic Materials USA Corp.

company:	70 Meister Avenue

Somerville, NJ 08876

Telephone No.: 800-515-4164

Information on the substance/preparation

Product Safety: 908-429-3562

Emergency Tel. number: 800-424-9300 CHEMTREC

Trade name:	AZ(R) 300 MIF DEVELOPER

Section 02 - Composition information

Hazardous ingredients:

|  | Chemical Name | CAS-no. (Trade | Concentration [%] |
| --- | --- | --- | --- |
|  |  | secret no.) |  |
|  | Tetramethylammonium hydroxide | 75-59-2 | 2.00 - 0.00 |
| Non-hazardous ingredients: |  |  |  |
|  | Chemical Name | CAS-no. (Trade | Concentration [%] |
|  |  | secret no.) |  |
|  | Water | 7732-18-5 | 98.00 |

Section 03 - Hazardous identification

Emergency overview: Clear liquid with slight amine odor., Noncombustible., Causes moderate skin irritation., Causes moderate eye irritation., Water soluble.

Expected route of entry

Skin contact:	Causes moderate skin irritation.

Ingestion:	May be harmful if swallowed.

Inhalation:	No hazard in normal industrial use.

Eye contact:	Causes moderate eye irritation.

Health effects of exposure:

Component information:

Eye: Causes severe eye irritation. Skin: Causes moderate skin irritation. Toxic by skin absorption.

Inhalation: No hazard in normal industrial use. Ingestion: May be harmful if swallowed. Reproductive and

birth defects: No information. Systemic effects: No hazard in normal industrial use.

1/6

MATERIAL SAFETY DATA SHEET

AZ(R) 300 MIF DEVELOPER

| Substance key: BBG70N4 | REVISION DATE: 07/25/2005 |
| --- | --- |
| Version | Print Date: 07/25/2005 |

Tetramethylammonium hydroxide (75-59-2)

Tetramethylammonium hydroxide may cause severe irritation or caustic burns to eyes and mucous membranes. TMAH is caustic and corrosive to skin and eyes in concentrated form. Pure TMAH is highly toxic in animal tests by the oral and dermal routes of exposure.

Listed carcinogen:	IARC: NO	NTP: NO	OSHA: NO

HMIS:

Health: 2	Flammability: 0	Reactivity: 0	Personal protection: X

NFPA:

Health: 2	Flammability: 0	Reactivity: 0	Special notice: NONE

Section 04 - First aid measures

After inhalation:	Remove victim to fresh air.

Consult physician if irritation occurs.

After contact with skin: Consult physician if exposure is extensive or if irritation occurs. Immediately remove contaminated clothing and wash affected area thoroughly with soap and water.

After contact with eyes: Flush thoroughly with water for 15 minutes. Get immediate medical help.

After ingestion: If person is conscious, give water or milk to dilute stomach contents.

Never give anything by mouth to an unconscious person. Consult physician.

Do not induce vomiting.

Advice to doctor / A component of this material causes severe acute toxicity in Treatement: experimental animals by the oral or dermal route of exposure.

Exposed individuals should be cafefully observed and treated according to symptoms.

Section 05 - Fire fighting measures

Flash point: Water-based material with low organic content., Compatible with extinguishing agents.

Section 06 - Accidental release measures

Steps to be taken in case of spill or leak:

Wearing appropriate personal protective equipment, contain spill, collect onto inert absorbent, and place in a suitable container. Rinse residual with water.

2/6

MATERIAL SAFETY DATA SHEET

AZ(R) 300 MIF DEVELOPER

| Substance key: BBG70N4 | REVISION DATE: 07/25/2005 |
| --- | --- |
| Version | Print Date: 07/25/2005 |

Section 07 - Handling and Storage

Advice on safe handling:

Use only with adequate ventilation and proper protective eyewear, gloves, and clothing.

Further information for storage conditions:

Store at appropriate temperature. See label for details.

Store in original container.

Keep from freezing.

Section 08 - Exposure Control / personal protection

Skin and body protection:	Clothing suitable to prevent skin contact.

Additional advice on Where mist is present, provide local exhaust ventilation or a system design: respirator certified for mist by NIOSH.

Section 09 - Physical and chemical properties

| Form: | Liquid |
| --- | --- |
| Color: | Clear, colorless |
| Odor: | Slight amine odor. |
| pH: | 13.3 |
| Water solubility: | soluble |
| Density: | 1 g/cm3 |
| Boiling point: | 100 °C |
| Loss on drying: | 97.6 % |

Section 10 - Stability and reactivity

Hazardous reactions:	Stable.

Hazardous polymerization:	Will not occur.

Conditions to avoid: Avoid contact with strong acids. This product is expected, by test or analogy, to slowly attack aluminum and perhaps other

3/6

MATERIAL SAFETY DATA SHEET

AZ(R) 300 MIF DEVELOPER

| Substance key: BBG70N4 | REVISION DATE: 07/25/2005 |
| --- | --- |
| Version | Print Date: 07/25/2005 |

nonferrous metals, releasing hydrogen gas.

Section 11 - Toxiclogical information

Acute oral toxicity: Testing in animals shows that this material is, harmful (rat acute oral LD50 between 500 and 5000mg/kg).

Tetramethylammonium hydroxide (75-59-2)

Acute oral toxicity:	LD50 rat

50 mg/kg as chloride salt

Tetramethylammonium hydroxide (75-59-2)

Acute inhalation toxicity	No data.

Tetramethylammonium hydroxide (75-59-2)

Acute dermal toxicity:	LD50 Guinea pig

25 mg/kg

not determined

Section 12 - Ecological information

Tetramethylammonium hydroxide (75-59-2)

Toxicity to fish:	LC50

35.1 mg/l

Tetramethylammonium hydroxide (75-59-2)

| Toxicity of aquatic | EC50 |
| --- | --- |
| invertebrates: | 0.21 mg/l |
| Tetramethylammonium hydroxide (75-59-2) |  |
| Toxicity to algae : | No data available. |

Section 13 - Disposal considerations

Product:	Consult local, state, and federal regulations.

This product would be considered a hazardous waste under

RCRA due to high pH unless neutralized prior to disposal.

Section 14 - Transport information

Land transport

- DOT:

4/6

MATERIAL SAFETY DATA SHEET

AZ(R) 300 MIF DEVELOPER

| Substance key: BBG70N4 |  | REVISION DATE: 07/25/2005 |
| --- | --- | --- |
| Version |  | Print Date: 07/25/2005 |
|  | Not restricted |  |
| Sea transport |  |  |
|  | IMDG: |  |
| UN-No: |  | 1719 |
| Proper technical name: |  | Caustic alkali liquid, n.o.s. contains (Tetramethylammonium |
|  |  | hydroxide, solution) |
| Class: |  | 8 |
| Packaging group: |  | III |
| Marine pollutant: |  |  |
| EmS: |  | F-A, S-B |
| MFAG: |  |  |
| Labels: |  | 8 |
| Air transport |  |  |
|  | ICAO/IATA-DGR: |  |
| UN/ID No.: |  | UN 1719 |
| Proper technical name: |  | Caustic alkali liquid, n.o.s. contains (Tetramethylammonium |
|  |  | hydroxide, solution) |
| Class: |  | 8 |
| Packaging group: |  | III |
| Labels: |  | 8 |

Section 15 - Regulatory information

| TSCA Status: | All components of this product are listed on the TSCA |
| --- | --- |
|  | Inventory. |
| SARA (section 311/312): | Reactive hazard: no |
|  | Pressure hazard: no |
|  | Fire hazard: no |
|  | Immediate/acute: yes |
|  | Delayed/chronic: no |
| SARA 313 information: | This product is not subject to SARA Title III Section 313 |
|  | reporting requirements under 40 CFR 372. |

Section 16 - Other information

Further information

5/6